- OBJECTIVES

See the requirements document

- HARDWARE DESIGN

See the PCB Artist schematic file

- SOFTWARE DESIGN

No change in software design (call graphs and data flow graphs are the same as those provided in the lab manual).

- MEASUREMENT DATA

4.1	Procedure 3 Data

| Digital Input | 0 | 512 | 1024 | 1536 | 2048 | 2560 | 3072 | 3584 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DAC Measured Output (V) | 0.07 | 0.43 | 0.81 | 1.19 | 1.57 | 1.93 | 2.3 | 2.68 |
| Expected Output (V) | 0 | 0.4125 | 0.825 | 1.2375 | 1.65 | 2.0625 | 2.475 | 2.8875 |
| Delta | 0.07 | 0.0175 | -0.015 | -0.0475 | -0.08 | -0.1325 | -0.175 | -0.2075 |

Range: 0 to 3.3V

Precision: 4096

Resolution: 0.81mV

Accuracy:

- ANALYSIS AND DISCUSSION

  - Briefly describe the three errors in a DAC.

i)  Offset Error: The difference between the DAC output and 0V when 0 is applied at the input.

ii)  Full-scale Error: The difference between ideal and actual DAC output when max input is applied. Very dependent on Vref stability.

iii)  Gain Error: Full-scale Error minus Offset Error. Deviation of input to output slope from ideal value.

  - Calculate the data available and data required intervals in the SSI/DAC interface. Use these calculations to justify your choice of SSI frequency.

  - How is the frequency range of a spectrum analyzer determined?

  - Why did we not simply drive the speaker directly from the DAC? I.e., what purpose is the TPA731?