- OBJECTIVES

See the requirements document

- HARDWARE DESIGN

See the PCB Artist schematic file

- SOFTWARE DESIGN

No change in software design (call graphs and data flow graphs are the same as those provided in the lab manual).

- MEASUREMENT DATA

  - Show the data and calculated resolution, range, precision and accuracy

Figure 1: Experimental measurement of the DAC output for 8 different digital inputs.

Our DAC’s actual output range is ~3 Volts (which makes sense because our

Vref  was 1.5 Volts. Our DAC seemed to output less voltage than expected for

most of the tested values.

| Digital Input; 0; 512; 1024; 1536; 2048; 2560; 3072; 3584; 4096. DAC Measured Output (V); 0.07; 0.43; 0.81; 1.19; 1.57; 1.93; 2.3; 2.68; 3.05. Expected Output (V); 0; 0.4125; 0.825; 1.2375; 1.65; 2.0625; 2.475; 2.8875; 3.3. Delta = Actual - Expected; 0.07; 0.0175; -0.015; -0.0475; -0.08; -0.1325; -0.175; -0.2075; -0.2467. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Range: 0 to 3.3V

Precision: 4096

Resolution: 0.81mV

Accuracy: 11.5%

4.2	Show the experimental response of DAC including SNR

4.3	Show the results of the debugging profile

Figure 1: This ISR shows the time it takes to output to the DAC (~1 microsecond)

Figure 2: This ISR shows the time it takes to change the tempo.

4.4	Measurements of current required to run the system, with and without the music playing

Figure 3: about 78 mA was required to run the system without music playing

Figure 4: about 90 mA was required to run the system with music playing

(this value did shoot up to 120-130 mA when playing louder music)

- ANALYSIS AND DISCUSSION

  - Briefly describe three errors in a DAC

i)  Offset Error: The difference between the DAC output and 0V when 0 is applied at the input.

ii)  Full-scale Error: The difference between ideal and actual DAC output when max input is applied. Very dependent on Vref stability.

iii)  Gain Error: Full-scale Error minus Offset Error. Deviation of input to output slope from ideal value.

  - Calculate the data available and data required intervals in the SSI/DAC interface. Use these calculations to justify your choice of SSI frequency

  - How is the frequency range of a spectrum analyzer determined?

  - Why did we not simply drive the speaker directly from the DAC? What is the purpose of using the TPA731?